Assignment - 4 Assignment Date; 22 October 2022. Student Name; ANN EDWIN JOSHWA.M. Student Roll Number; 95071914010. Maximum Marks; 2 Marks. Question-1: Write code and connections in wokwi for ultrasonic sensor.Whenever distance is less than 100 cms send “Alert” to ibm cloud and display in device recent events. Solution: // Pins const int TRIG_PIN = 7; const int ECHO_PIN = 8; // Anything over 400 cm (23200 us pulse) is "out of range" const unsigned int MAX_DIST = 23200; void setup() { // The Trigger pin will tell the sensor to range find pinMode(TRIG_PIN, OUTPUT); digitalWrite(TRIG_PIN, LOW); //Set Echo pin as input to measure the duration of //pulses coming back from the distance sensor pinMode(ECHO_PIN, INPUT); // We'll use the serial monitor to view the sensor output Serial.begin(9600); } void loop() { unsigned long t1; unsigned long t2; unsigned long pulse_width; float cm; float inches; // Hold the trigger pin high for at least 10 us

digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10); digitalWrite(TRIG_PIN, LOW); // Wait for pulse on echo pin while ( digitalRead(ECHO_PIN) == 0 ); // Measure how long the echo pin was held high (pulse width) // Note: the micros() counter will overflow after ~70 min t1 = micros(); while ( digitalRead(ECHO_PIN) == 1); t2 = micros(); pulse_width = t2 - t1; // Calculate distance in centimeters and inches. The constants // are found in the datasheet, and calculated from the assumed speed //of sound in air at sea level (~340 m/s). cm = pulse_width / 58.0; inches = pulse_width / 148.0; // Print out results if ( pulse_width > MAX_DIST ) { Serial.println("Out of range"); } else { Serial.println("*******************************"); Serial.print("The Measured Distance in cm : "); Serial.println(cm); if(cm<100){ // while(true){ Serial.println("Alert!!"); // } } Serial.print("*******************************"); } // Wait at least 1000ms before next measurement delay(1000); }

Project Link: https://wokwi.com/projects/new/arduino-uno.